USER’S MANUAL

Thermal Receipt Printer

TP-100

Shandong New Beiyang Information Technology Co., Ltd.

Declaration

Information in this document cannot be changed without notice. SHANDONG NEW BEIYANG INFORMATION TECHNOLOGY CO., LTD. (hereinafter referred to as “SNBC”) reserves the right to improve products as new technology, components, software, and firmware become available. If users need further data about this product, please feel free to contact SNBC or your local dealer.

No part of this document may be reproduced or transmitted in any form by any means, electronic or mechanical, for any purpose without the express written permission of SNBC.

Copyright

Copyright © 2017 by SNBC

Printed in China

Version 1.0

Warning and Caution

Warning: Items shall be strictly followed to avoid injury or damage to body and devices.

Caution: Items with important information and prompts for operating the printer.

General Safety Information

Before installing and using the printer, please read the following items carefully.

- Safety instructions

Warning: Do not touch the cutter of the printer.

Warning: The print head is a thermal element and it is at a high temperature during printing or just after operation, therefore do not touch it or its peripherals for reasons of safety.

Warning: The print head is an ESD-sensitive device. To avoid damage, do not touch either its printing parts or connecting parts.

- Caution

- Install the printer on a flat and stable place.

- Reserve adequate space around the printer so that convenient operation and maintenance can be performed.

- Keep the printer away from water source, and avoid direct exposure to sun light, strong light and heat of fire.

- Do not use or store the printer in a place exposed to high temperature, moisture and serious pollution.

- Do not place the printer on a place exposed to vibration or impact.

- No dew condensation is allowed to the printer. In case of such condensation, do not turn on the power until it has completely evaporated.

- Connect the DC adapter to an appropriate grounding outlet. Avoid sharing a single electrical with large power motors and other devices that may cause the fluctuation in voltage.

- Disconnect the DC adapter when the printer is not used for a long time.

- Do not let the water or electric materials (like metal) go into the printer. If this happens, turn off the power immediately.

- Do not allow the printer to start printing when there is no recording paper installed; otherwise the print head and platen roller will be seriously damaged.

- To ensure the print quality and normal lifetime, use recommended or good quality paper.

- Shut down the printer when connecting or disconnecting interfaces connectors to avoid damage to the control board.

- Set the print darkness to a lower grade as long as the print quality is acceptable. This will help to keep the print head durable.

- The printer should only be disassembled or repaired by a technician, who is certified by the manufacturer.

- Keep this manual safe and at hand for ready reference.

Caution: The following content about cutter only applies to printers with cutter.

Contents

1	Introduction	- 1 -

1.1	Outline	- 1 -

1.2	Features	- 1 -

2	Specification	- 2 -

2.1	Technical specification	- 2 -

2.2	Cutter parameters	- 3 -

2.3	Paper specification	- 3 -

2.3.1	Parameters of continuous paper	- 3 -

2.3.2	Parameters of marked paper	- 4 -

3	Appearance and components	- 5 -

3.1	Appearance and modules	- 5 -

3.2	LED and buzzer	- 7 -

4	Installation	- 8 -

4.1	Unpacking	- 8 -

4.2	Printer installation	- 8 -

4.3	Power and communication interface	- 9 -

4.3.1	Power connection	- 9 -

4.3.2	Interface connection	- 9 -

4.3.3	USB interface connection	- 10 -

4.3.4	Cash drawer connection	- 10 -

4.4	Paper roll installation	- 10 -

4.4.1	Paper type confirmation	- 10 -

4.4.2	Install/replace paper roll	- 10 -

4.5	Paper near end position adjustment	- 13 -

4.6	Power-on and self-test	- 14 -

4.6.1	Power-on	- 14 -

4.6.2	Print self-test page	- 14 -

4.7	Hexadecimal dumping function	- 14 -

5	Routine maintenance	- 16 -

5.1	Cleaning the print head and the platen roller	- 16 -

5.2	Cleaning the mark sensor	- 16 -

5.3	Clearing the jammed paper and the cutter error	- 16 -

6	Interface signal	- 17 -

6.1	Serial interface	- 17 -

6.2	Parallel interface	- 17 -

6.3	USB interface	- 19 -

6.4	Ethernet interface	- 19 -

6.5	Double communication interface	- 20 -

6.6	Power interface definition	- 21 -

6.7	Cash drawer interface signal definition	- 21 -

7	Troubleshooting	- 23 -

7.1	Printer doesn’t work	- 23 -

7.2	Error LED and buzzer alarm	- 23 -

7.3	Problems during printing	- 23 -

8	Power management	- 24 -

9	DUMP mode	- 25 -

Appendix Button configuration	- 26 -

Introduction

Outline

BTP-S80 is developed for high-end thermal receipt printing market, which is widely used.BTP-S80 can be connected with other devices via serial, parallel, USB, Bluetooth, Ethernet and WLAN. It provides drivers for operation systems such as Windows 2000 / XP /Server 2003 / Vista / Server200 / Win7 / Win8 /Win8.1 / Win10/Server 2012, POSReady2009 / POSReady7, Linux, Mac and UPOS middleware.

Note: Windows and Linux drivers support 64-bit operation system.

Features

- Low noise, high printing speed;

- Support continuous paper, marked paper;

- Support paper saving, water mark, upside-down, two-color printing, etc.;

- Compact size, can output paper from the front or the top according to users’ different demands;

- Rich interfaces: USB(fix on board), expandable serial / parallel / Ethernet / Bluetooth / WIFI / serial + Ethernet interface;

Specification

Technical specification

| Item |  |  |  | Parameter |
| --- | --- | --- | --- | --- |
| Printing |  | Print method |  | Direct thermal line printing |
|  |  | Resolution |  | 203 × 203DPI; 203 × 180DPI |
|  |  | Paper width |  | 57.5/80/82.5mm |
|  |  | Print width |  | Max.80mm |
|  |  | Print speed |  | Max.250mm/s |
|  |  | Flash LOGO |  | Max. 1M bytes |
|  |  | Command buffer area |  | 64K bytes, 4K bytes or 45 bytes |
| Interface |  | USB |  | USB2.0 (full speed) |
|  |  | Serial interface |  | RS-232C |
|  |  | Parallel interface |  | IEEE1284 |
|  |  | Ethernet interface |  | 10/100BASE-T |
|  |  | Cash drawer |  | Can select to controlling 1～2 cash drawers |
| Memory |  |  |  | RAM: 2MB, Flash: 4MB |
| Printer status detection |  |  |  | Paper end sensor/ black mark sensor; Paper near end sensor; Cover position sensor; Paper presence sensor Print head temperature sensor; Printer voltage detection |
| Cut mode |  |  |  | Full cut, partial cut |
| Barcode |  | 1D |  | UPC-A, UPC-E, CODE 39, CODE 93, CODE 128, EAN8, EAN13, ITF, CODABAR |
|  |  | 2D |  | PDF417, QR, Maxicode |
| Fonts |  |  |  | Font A: 12 × 24 Font B: 9 × 17 Kanji font A: 24 × 24 |
| Character set |  |  |  | 95 Alphanumeric |
|  |  |  |  | 14 types of international characters |
|  |  |  |  | 128 x 68 code page |
|  |  |  |  | Optional: Traditional Chinese, Simplified Chinese (GB2312/18030), Japanese, Korean, English, HK |
|  |  |  |  | User-defined font (95) & code page |
| Character enlargement |  |  |  | All characters can be enlarged 1-6 times horizontally and vertically |
| Character rotation |  |  |  | Rotation printing in four directions (0°, 90°, 180°, 270°) |
| Command |  |  |  | ESC/POS compatible |
| Paper | Paper type |  |  | Thermal continuous paper Thermal marked paper |
|  | Paper roll OD |  |  | Max. 83mm |
|  | Paper thickness |  |  | 0.06mm~0.10mm thermal paper |
| Power supply | Input |  |  | 100-240VAC, 50-60Hz |
|  | Output |  |  | 24V ± 5% DC, average current 2A |
|  | External or not |  |  | External power adapter |
| Human-machine interface |  |  | Power switch | Support |
|  |  |  | Button | Support |
|  |  |  | LED | Support, including POWER LED and ERROR LED |
|  |  |  | Buzzer | Support, 24V buzzer |
| Reliability |  | Lifetime of print head |  | ≥150Km (standard test sample with 12.5% duty ratio) |
|  |  | Lifetime of cutter |  | 2,000,000 cuts (standard test condition) |
|  |  | MCBF |  | 70,000,000 lines |
|  |  | MTBF |  | 360,000 hours (main control board) |
| Operation temperature and humidity |  |  |  | 5°C～45°C，20%～90%RH(40°C) |
| Storage temperature and humidity |  |  |  | -40°C～60°C，10%～90%RH(40°C) |
| Overall dimensions |  |  |  | 127mm(L)*127mm(W)*134mm(H) |
| Functions |  | Saving paper |  | Support |
|  |  | Button configuration |  | Support (configure the printer without computer) |
|  |  | Right-up-side printing |  | Support |
|  |  | Water mark printing |  | Support |
|  |  | Gray scale printing |  | Support |

Table 2.1-1 Technical specifications

Cutter parameters

| Item | Parameter | Remarks |
| --- | --- | --- |
| Cutting method | Sliding blade |  |
| Cutting time | 500ms | The time of one cut |
| Cutting interval | 3s | 20 cuts/minute (Max.) |
| Applicable paper types | 0.06mm~0.10mm thermal paper |  |
| Operation voltage | 24VDC |  |
| Max. operation current | 1.2A | Operation voltage 24VDC |
| Cutter lifetime | 2,000,000 cuts | Standard test condition |

Table 2.2-1 Technical parameters of cutter

Paper specification

Parameters of continuous paper

- Paper type: thermal paper

- Paper supply mode: paper roll

- Paper width: 57.5/80/82.5mm

- Paper thickness: 0.06mm-0.10mm

- Thermal layer: outward

- Paper roll specification: paper roll OD Max ф83 mm, core ID Min. ф12.8mm

- Paper recommended:

| Paper model | Manufacturer |
| --- | --- |
| 600-3.1 | APPLETON Papers Inc |
| KLS_46_e | KANZAN Spezialpapiere GmbH |
| FD210 | OJI Paper CO., LTD. |

Table 2.3-1 Recommended thermal paper model

Parameters of marked paper

BTP-S80 can support marked paper printing and set the cutting and initial printing position accurately. The marked paper not only should meet the specifications of thermal paper roll, but also should meet the following requirements:

- L1 mark height: 3mm≤ L1≤13mm

- L2 mark length: L2≥8mm

- L3 distance between two marks: 30mm≤L3<450mm

- Thermal side mark sensor position: reserved the left/middle/right positions

- Non-thermal side mark sensor position: the middle/left/right position is selectable

- Reflectivity: The reflectivity of the black mark must be no more than 15% while the reflectivity of the paper itself should exceed 85%. There should be no image between the two marks, such as the advertisement, etc.

Caution:

    - Please use the recommended paper or its equivalents. Using the paper of low quality might affect the print quality and shorten the lifetime of print head;

    - Do not stick paper onto the core shaft;

    - If the paper is contaminated by chemical or oil, it may discolor or lose heat sensitivity at the polluted spot, which will affect the print affect;

    - Do not rub the paper surface with hard objects, otherwise it may affect the print affect;

    - When the environment temperature goes up to 70℃, paper will discolor. Thus don not use or store paper under high temperature, high humidity and strong light conditions.

    - The mark is measured during printing and paper feeding. If the mark height value detected by the sensor is bigger than the default value (default setting is 13mm), the printer will alarm paper end.

Appearance and components

Appearance and modules

Fig. 3.1-1 Schematic drawing of appearance and modules

| 1—Top cover | 2—Cutter | 3—Platen roller |
| --- | --- | --- |
| 4—Paper end sensor | 5—Paper guide | 6—Micro switch |
| 7—Cutter cover | 8—Middle cover | 9—Cover open spanner |
| 10—Paper cabinet | 11—Paper near end sensor | 12—FEED button |
| 13—ERROR LED | 14—POWER LED | 15—POWER button |
| 16—USB | 17—Power interface | 18—Communication interface |
| 19—Cash drawer interface |  |  |

Button and component function:

a	FEED button (12)

- Feed paper:

Printer will feed paper when the feed button is pressed down under normal condition. To feed paper continuously, keep pressing the button.

- Print configuration sample:

Pressing down the feed button while turning on the power, the printer will print out the configuration sample, which includes print width, print speed, etc.

- Enter button configuration mode:

Press down the feed button while turning on the power, the printer will print out the configuration sample, and enter pause status (error LED flashes) after cutting paper. Keep pressing the feed button at this time, the printer will enter button configuration mode.

- Press the button to clear the cutter error:

Press the button for a short time when a cutter error occurs, the printer will try to clear the cutter error automatically.

b	Error LED (13)

Indicate printer status. Under normal status, the error LED is off. Under error status (e.g. paper end, etc.), the error LED flashes.

c	Power LED (14)

Indicate printer power status (ON/OFF).

d	Power button (15)

Press the button to connect power; press the button for a long time to disconnect the power.

e	Top cover status detection sensor (14)

Detect the printer cover status (open/closed).

f	Paper presence sensor (12)

Detect the paper presence / absence status when continuous paper is used; detect the paper marks when marked paper is used.

g	Spanner for adjusting paper near end sensor(7)

Detect the paper roll status. Fast flashing of error LED indicates that the paper will be used out soon and the user should replace the paper roll in time. Printer will work normally until paper is used out.

h	Paper guide (5)

Adjust the position of paper guide in the cabinet to adapt to different paper widths between 58/80mm. When the paper guide is removed, the printer can adapt to 82.5mm paper width.

LED and buzzer

- Functions of LED

| LED name | Status | Description |
| --- | --- | --- |
| Power LED (green) | Always on | Printer power is on. |
|  | Off | Printer power is off. |
| Error LED (red) | Off | Printer is in normal status. |
|  | Flash | Printer is in error status or paper near end status. |
| Buzzer | Beep | Printer is in error status. |

Table 3.2-1 Functions of LED

- Error type indicated by LED & buzzer

| Error Type | Error LED | Buzzer |
| --- | --- | --- |
| Print head is overheating | Cycle flash 6 times | Cycle beep 6 times |
| Printer voltage is abnormal | Cycle flash 5 times | Cycle beep 5 times |
| Cutter error | Cycle flash 4 times | Cycle beep 4 times |
| Print head lift-up | Cycle flash 3 times | Cycle beep 3 times |
| Paper end | Cycle flash 2 times | Cycle beep 2 times |
| Paper near end | Cycle flash slowly | No beep |
| Cannot find mark or mark calibration error | Cycle flash slowly | No beep |

Table 3.2-2 LED and indication information

Caution:

- The temperature of the print head is detected by a thermal resistor. If the print head is overheating, the protective circuit will shut off the power automatically and force the printer to stop printing; the temperature of print head when printing is stopped is 65℃.

Installation

Unpacking

Check whether all items, which are listed on the packing list, are present and in a good condition. If any item is damaged or missing, please contact your dealer or the manufacturer.

Printer installation

- BTP-S80 supports two working methods: out put the paper from the front or the top when place the printer horizontally on the table. When the printer is placed horizontally on the table, the incline installation angle should not exceed 5°, otherwise the paper near end sensor will not work normally.

Fig. 4.2-1 Horizontally on the table (paper output from the front)

Fig. 4.2-2Horizontally on the table (paper output from the top)

- Keep the printer far away from water source;

- Do not place the printer in the place exposed to vibration and impact;

- The printer power must be safely grounded;

- It is recommended to keep proper space in order to guarantee the reliability and operational convenience of the printer during operation and maintenance.

Fig.4.2-3Printer maintenance and operation space

Power and communication interface

Power connection

- Ensure that the power switch is turned off;

- Insert the power plug into the corresponding socket on the back of the printer.

Caution:

- When the printer is not in use for a long period of time, disconnect the power cord from the printer.

Interface connection

- Ensure that the power switch is turned off;

- Plug the connector into corresponding port and fix it with screw or spring as shown in the figure;

- Connect the other end of cable to the host.

USB interface connection

- Ensure that the power switch is turned off;

- Plug the USB cable into corresponding interface in place as shown in the figure;

- Connect the other end of USB cable to the host.

Cash drawer connection

- Ensure that the power switch is turned off;

- Plug the cash drawer cable into the cash drawer interface which locates at the back of printer.

Caution:

- The cash drawer interface is used to connect the cash drawer only (cannot connect with phone wire, etc.).

Paper roll installation

Paper type confirmation

After connecting the power cord and interface cable, confirm paper type after media installation before printing.

Install/replace paper roll

- Turn off the printer;

- Push the spanner in the direction shown in the Fig. to open the top cover;

- Place the paper roll into the printer;

- Close the top cover.

Fig. 4.4-1 Schematic drawing of paper roll installation/replacement

Fig. 4.4-2 Schematic drawing of paper roll installation/replacement

图8

图7

图7

Caution:

- Adjust the paper guide according to the paper specification: install the paper guide at the position of 57.5 for 57.5 wide paper roll; install the paper guide at the position of 80 for 80 wide paper roll; and remove the paper guide for 82.5 wide paper roll. Pay attention that the winding direction of paper roll should meet printer requirements. Remove the end in operation hole firstly when disassemble the paper guides, and install the end in operation hole firstly when install the paper guides;

Fig. 4.4-3 Failure model

- Ensure that the paper roll is tightly wound; otherwise paper jam or other failure may occur (eg. Fig.4-3 Failure model );

- Paper roll should be placed stably in the paper cabinet without incline, or the printing will be affected.

Fig.4.4-4 Position of 57.5 Fig. 4.4-5 Position of 80

Fig. 4.4-5 Operation hole

Paper near end position adjustment

Fig. 4.5-1 Schematic drawing of paper near end position adjustment (output paper from the front)

Fig. 4.5-2 Schematic drawing of paper near end position adjustment (output paper from the front)

The following diameter of core shaft of paper roll supported by the printer are as following: OD of 16.2mm core shaft and OD of 20.8mm core shaft; adjust the paper near end sensor to adjust the application method and paper near end alarm function for core shaft with different outer diameters. There are four positions indicated by the scale marks for paper near end alarm, and users can rotate the paper near end adjustment spanner to make the indication line locate at different position, which change the paper near end alarm functions under different application conditions. The position 1 and 2 are for the application that output paper from the top, position 1 is for 16.2mm core shaft of paper roll and position 2 is for 20.8mm core shaft of paper roll. The position 3 and 4 are for the application that output paper from the front, position 3 is for 20.8mm core shaft of paper roll and position 4 is for 16.2mm core shaft of paper roll.

Fig. 4.5-3 core shaft of paper roll

Power-on and self-test

Power-on

- Ensure that the printer is connected to power;

- Turn on the power switch to power on the printer.

Print self-test page

- Ensure that the printer is connected to power, and that paper roll is installed;

- Ensure that the power LED is off and the printer is under power-off condition.

- Press the feed button while turning on the printer power, and then release the button. The printer will print out configuration information and prompt characters “Press and Release FEED key to print characters” and “Press and Hold FEED key to config the printer”. Then the printer enters pause status, and the error LED flashes;

- Pressing the feed button momentarily, the printer will print out a character test sample, and the printing of self-test page is completed. Pressing the feed button for a long time, the printer will enter button configuration mode.

Hexadecimal dumping function

After entering Hexadecimal dumping mode, the printer will print out the data transmitted from the host computer in hexadecimal and their corresponding ASCII characters.

The sample printed under Hexadecimal dumping mode is as follows:

Fig. 4.7-1 Print sample under Hexadecimal dumping mode

Using Hexadecimal dumping mode:

- Entering Hexadecimal dumping mode in the following ways:

- Open the printer mechanism and turn on the printer while pressing the feed button. Release the button after the printer alarms (LED flashes and buzzer beeps).

- Send command “GS  ( A”.

- The printer first prints "Hexadecimal Dump To terminate …..", and then prints the data transmitted from the host in hexadecimal and their corresponding ASCII characters.

- Exiting from Hexadecimal dumping mode in the following ways:

- Turn off the power, and then restart the printer.

- Press the feed button three times.

图8

图7

图7

Caution:

- If the hexadecimal data has no corresponding ASCII characters, the printer will print ".";

- Under Hexadecimal dumping mode, only commands DLE EOT, DLE ENQ, or DLE DC4 are valid.

- The data of the last character line can be printed by pressing down the feed button.

Routine maintenance

Caution:

- Before starting routine maintenance, ensure that the printer power is turned off.

- Do not use organic solvents like gasoline or acetone.

- When cleaning sensors, do not turn on the printer power until the pure alcohol has completely evaporated.

- It is recommended that the maintenance cycle should not be longer than one month.

Cleaning the print head and the platen roller

Follow the steps below to clean the print head and the platen roller:

- Turn off the printer and open the top cover;

- If the printing was just finished, please wait for the print head to cool down completely;

- Wipe off the dust and stains on the surface of print head and platen roller with alcohol cotton (it should be wrung out);

- After the alcohol is completely evaporated, close the top cover.

Cleaning the mark sensor

When the printer cannot identify the mark effectively, the mark sensor should be cleaned.

The cleaning steps are as follows:

- Turn off the printer;

- Press the cover open lever to open the top cover;

- Wipe off the dust and stains on the surface of the sensor with soft cotton cloth dipped with pure alcohol (it should be wrung out);

- Install the sensor cover after the pure alcohol is completely evaporated, then close the top cover and finish mark sensor cleaning.

Clearing the jammed paper and the cutter error

Steps for clearing jammed paper and cutter error are as follows:

Method 1:

- Turn off the printer power and turn the spanner to open the top cover;

- Clear the jammed paper, and then close the top cover;

- Turn on the printer power again, and then the cutter can be reset automatically.

Method 2:

- Turn off the printer power, disassemble the cutter cover in the direction shown in the figure, and turn the spanner to open the top cover;;

- Clear the jammed paper, close the paper cabinet and turn on the printer power again, and then the cutter can be reset automatically.

- Install the cutter cover.

Interface signal

Serial interface

The serial interface of the printer is compatible with RS-232 standard, and its interface socket is D-SUB25 socket.

| Pin | Signal | Signal definition | Function |
| --- | --- | --- | --- |
| 1 | FG | — | House ground |
| 2 | TXD | Output | Data output |
| 3 | RXD | Input | Data input |
| 5 | NC | — | Not connected |
| 6 | DSR | Input | Data device is ready |
| 7 | SG | — | Signal ground |
| 8－19 | NC | — | Not connected |
| 4,20 | DTR | Output | Require to send |
| 21－25 | NC | — | Not connected |

Fig. 6.1-1 Serial interface definition

User can query interface setting status via printing configuration sample. The default settings of serial interface are as follows:

Baud rate: 115200bps; Data bits: 8; Parity: none; Stop bit: 1; Handshake: DTR/DSR.

Parallel interface

The parallel interface can work in IEEE 1284 compatible mode or nibble mode. The interface is 36PIN socket.

Interface definition:

| Pin No. | Signal source | Signal definition |
| --- | --- | --- |
| 1 | H | nStrobe |
| 2 | H | Data 0 (Least Significant Bit) |
| 3 | H | Data 1 |
| 4 | H | Data 2 |
| 5 | H | Data 3 |
| 6 | H | Data 4 |
| 7 | H | Data 5 |
| 8 | H | Data 6 |
| 9 | H | Data 7 (Most Significant Bit) |
| 10 | P | nAck |
| 11 | P | Busy |
| 12 | P | Perror |
| 13 | P | Select |
| 14 | H | nAutoFd |
| 15 |  | Not defined |
| 16 |  | Logic Gnd |
| 17 |  | Chassis Gnd |
| 18 | P | Peripheral Logic High |
| 19 |  | Signal Ground (nStrobe) |
| 20 |  | Signal Ground (Data 1) |
| 21 |  | Signal Ground (Data 2) |
| 22 |  | Signal Ground (Data 3) |
| 23 |  | Signal Ground (Data 4) |
| 24 |  | Signal Ground (Data 5) |
| 25 |  | Signal Ground (Data 6) |
| 26 |  | Signal Ground (Data 7) |
| 27 |  | Signal Ground (Data 8) |
| 28 |  | Signal Ground (PError, Select, and nAck) |
| 29 |  | Signal Ground (Busy and nFault) |
| 30 |  | Signal Ground (nAutoFd, nSelctIn, and nInit) |
| 31 | H | nInit |
| 32 | P | nFault |
| 33 |  | Not defined |
| 34 |  | Not defined |
| 35 |  | Not defined |
| 36 | H | nSelectIn |
| 36 | H | nSelectIn |

- H stands for the host and P stands for the printer;

- The letter “n” in front of signal name indicates that the low level is effective;

- In data transmission, the host should not ignore “Busy” signal; otherwise print data may be missing;

- If the host cannot provide all the signal lines listed in the table above, the communication may fail;

- Parallel interface signal adopts TTL level; the rise and fall time of the signal from the host must be controlled within 0.5s when it is used;

- For the interface, the signal line should use twisted pair with feeder line end, with the feeder line end connected to signal ground;

- The parallel interface connecting wire should be as short as possible on condition that it meets the use condition.

USB interface

- Parameters

Data transmission: Support USB2.0 full speed protocol.

Connector (Printer end): USB B type socket. Support USB HUB.

- Interface signal definition and function

| Pin No. | Signal name | Description |
| --- | --- | --- |
| 1 | VBUS | Power |
| 2 | DATA- | Data minus |
| 3 | DATA+ | Data plus |
| 4 | GND | Ground |

Table 6.3-1 USB interface definition

- Interface connector

Fig. 6.2-1 USB interface connector

- Mode

USB interface can work under API mode and Windriver mode. API mode is the manufacturer self-defined mode, and the user needs to install the USB driver provided by the manufacturer. Windriver mode is also called class mode, it is no need to install drivers and the user can use Microsoft's generic driver. The factory setting of printer is API mode, the user needs to adjust it to class mode if the printer is going to work under a Linux system.

Ethernet interface

- Interface features

  - Support 10/100BASE-T communication standard

  - Compatible with Ethernet II standard frame type

  - LED shows network connection status and data transmission status

  - Support 9100 port print

  - Support status back

  - Support parameter configuration

  - Support firmware on-line upgrade

  - Support printer status query and interface module maintenance based on HTTP

- Interface signal definition

Fig.6.4-1 Ethernet interface

Interface adopts 10/100BASE-T standard which complies with IEEE802.3. The interface signal is defined as below:

| Pin No. | Signal name | Description |
| --- | --- | --- |
| 1 | TX+ | Data transmission + |
| 2 | TX- | Data transmission - |
| 3 | RX+ | Data receiving + |
| 4 | NC | Not connected |
| 5 | NC | Not connected |
| 6 | RX- | Data receiving - |
| 7 | NC | Not connected |
| 8 | NC | Not connected |

Table 6.4-1Ethernet interface definition

Double communication interface

Double communication interface support serial interface and Ethernet interface at the same time.

- The serial interface of the printer is compatible with RS-232 standard, and its interface socket is DSUB-9 socket. Pin definition is as following

| PIN No. | Signal definition |
| --- | --- |
| PIN1 | Not connected |
| PIN2 | RXD |
| PIN 3 | TXD |
| PIN 4 | DTR |
| PIN 5 | SG |
| PIN6 | DSR |
| PIN 7 | RTS |
| PIN 8 | CTS |
| PIN 9 | Not connected |

- Ethernet interface

Details refer to section “6.4 Ethernet interface”.

Power interface definition

- Power interface signal definition (100-240V AC):

Fig.6.4-1 Schematic drawing of power interface

| Pin No. | Signal name |
| --- | --- |
| 1 | E |
| 2 | L |
| 3 | N |

Table 6.6-1 Power interface definition

- Main control board power interface definition (24V DC):

Fig.6.6-2 Schematic drawing of 24V power interface

Cash drawer interface signal definition

- Electrical features

  - Driving voltage: DC 24 V

  - Driving current: Max. 1 A

  - Cash drawer status detection signal: “L” = 0～0.5 V “H” = 3.3 V

- Cash drawer interface socket uses RJ-11 6P connector

Fig.6.7-1Schematic drawing of cash drawer interface

- Interface signal definition

| No. | Signal | Functions |
| --- | --- | --- |
| 1 | FG | Frame Ground |
| 2 | DRAWER 1 | Cash drawer 1 driving signal |
| 3 | DRSW | Cash drawer status detection signal |
| 4 | VDR | Cash drawer driving power |
| 5 | DRAWER 2 | Cash drawer 2 driving signal |
| 6 | GND | Circuit share ground |

Table 6.7-1Cash drawer interface definition

Caution:

- Do not connect or disconnect communication cable plug when the printer is powered on;

- Communication cable should be far away from strong current;

- Communication cable should adopt shielded cable.

Troubleshooting

When the printer has any problem, refer to this chapter for solution.

If the problem still cannot be solved, please contact your local dealer or manufacturer for assistance.

Printer doesn’t work

| Problem | Possible causes | Solution |
| --- | --- | --- |
| LED is off and the printer doesn’t work. | Printer is not connected to power supply. | Connect the printer to power supply. |
|  | Printer power is off. | Turn on the printer power. |
|  | Circuit board is damaged. | Contact your local dealer or manufacturer. |

Table 7.1-1 Troubleshooting of printer not working

Error LED and buzzer alarm

| Problem | Possible causes | Solution |
| --- | --- | --- |
| Error LED flashes or buzzer beeps. | Paper end | Replace paper roll. |
|  | Cutter error | Clear cutter error. |
|  | The top cover is open | Close the top cover. |
|  | Print head is overheating. | Turn off printer power and wait for the print head to cool down. |
| Error LED is always on and buzzer keeps beeping. | The printer has serious malfunction. | Contact your local dealer or the manufacturer. |

Table 7.2-1 Troubleshooting of error LED and buzzer alarm

Problems during printing

| Problem | Possible causes | Solution |
| --- | --- | --- |
| Paper cannot be sent out normally. | Paper jam | Open the top cover, check paper path and cutter, and clear paper jam. |
| Printer starts printing but stops suddenly during printing. | Paper jam | Open the top cover, check the cutter, and clear paper jam. |
| Paper is not cut off. | Paper jam | Open the top cover, check the cutter, and clear the jammed paper. |
| Printout is not clear or has stains. | Paper roll is not installed correctly. | Check whether the paper roll is installed correctly or not. |
|  | Paper is out of specification. | Use the recommended thermal paper. |
|  | Dirty print head or platen roller | Clean the print head or the platen roller. |
|  | Print darkness is too low. | Increase the print darkness as needed. |
| Vertical print content is missing. | Dirty print head or platen roller | Clean the print head or the platen roller. |
|  | Print head error | Contact your local dealer or manufacturer. |

Table 7.3-1 Troubleshooting during printing

Power management

The power management of BTP-S80 has four operation modes: Off, Standby, Active, Sleep.

Printer will enter standby mode after powering on the printer or completing printing task.

Printer will enter sleep mode if there is no printing task for 2 minutes in standby mode.

Printer will awake automatically and enter active mode when a printing task comes, and will enter standby mode again after completing the printing task.

DUMP mode

There are two methods to enter DUMP mode:

- Command method: Send 1D 28 41 02 00 00 01

- Manual mode: Open the top cover, press and hold the FEED button, and turn on the printer power, then close the paper cabinet and release the button after the ERROR LED flashes. Pay attention to keep paper in the paper cabinet.

The printer will print out the following contents after the above operation:

“Hexadecimal Dump”

“To terminate hexadecimal dump, press FEED button three times”

The printer already entered the DUMP mode at this moment.

Method to exit from the DUMP mode: continuously press the FEED button for three times, and the printer will print out the following contents:

"***completed***"

The printer already exited from the DUMP mode at this moment.

- Appendix Button configuration

| PARAMETERSETTINGBYFEEDBUTTON |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAINMENU |  |  |  |  |  |  |  |  |  |
| Exit | ->1 |  |  |  |  |  |  |  |  |
| Print SelfTest | ->2 |  |  |  |  |  |  |  |  |
| Configuration | ->3 | CONFIGURATION |  |  |  |  |  |  |  |
|  |  | ExitWithoutSave | ->1 |  |  |  |  |  |  |
|  |  | ExitWithSave | ->2 |  |  |  |  |  |  |
|  |  | Communication | ->3 | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | USBInterface | ->2 | USBMODE:APIMODE |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | WinDriverMode | ->2 |  |  |
|  |  |  |  |  |  | APIMode | ->3 |  |  |
|  |  |  |  | Serial Interface | ->3 | SERIALINTERFACE |  |  |  |
|  |  |  |  |  |  | Back To Last Menu | ->1 |  |  |
|  |  |  |  |  |  | Baud Rates | ->2 | BAUD RATES:115200bps |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | 9600bps | ->2 |
|  |  |  |  |  |  |  |  | 19200bps | ->3 |
|  |  |  |  |  |  |  |  | 38400bps | ->4 |
|  |  |  |  |  |  |  |  | 57600bps | ->5 |
|  |  |  |  |  |  |  |  | 4800bps | ->6 |
|  |  |  |  |  |  |  |  | 2400bps | ->7 |
|  |  |  |  |  |  |  |  | 1200bps | ->8 |
|  |  |  |  |  |  |  |  | 115200bps | ->9 |
|  |  |  |  |  |  | Parity | ->3 | PARITY:NONE |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | None | ->2 |
|  |  |  |  |  |  |  |  | Odd | ->3 |
|  |  |  |  |  |  |  |  | Even | ->4 |
|  |  |  |  |  |  | DataBits | ->4 | DATABITS: 8Bits |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | 7Bits | ->2 |
|  |  |  |  |  |  |  |  | 8Bits | ->3 |
|  |  |  |  |  |  | StopBit(s) | ->5 | STOPBITS: 1Bit |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | 1Bit | ->2 |
|  |  |  |  |  |  |  |  | 2Bits | ->3 |
|  |  |  |  |  |  | Handshaking | ->6 | HANDSHAKING:DTR/DSR |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | DTR/DSR | ->2 |
|  |  |  |  |  |  |  |  | XON/XOFF | ->3 |
|  |  |  |  |  |  | DataReceive Error | ->7 | DATAERRORSETTING:Ignored |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | Ignored | ->2 |
|  |  |  |  |  |  |  |  | Print '?' | ->3 |
|  |  |  |  | Rx Buff Size | ->4 | RX BUFFER SIZE:4K Bytes |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | 4k Bytes | ->2 |  |  |
|  |  |  |  |  |  | 45Bytes | ->3 |  |  |
|  |  |  |  |  |  | 64KBytes | ->4 |  |  |
|  |  | Mechanism &Hardware | ->4 | HARDWARESETTINGS |  |  |  |  |  |
|  |  |  |  | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | Mark Sensor | ->2 | MARKSENSOR: Disable |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Enable | ->2 |  |  |
|  |  |  |  |  |  | Disable | ->3 |  |  |
|  |  |  |  | Cutter | ->3 | CUTTER: |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Cut Mode Settings | ->2 | CUTMODESETTINGS: DefaultCutMode |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | Enable | ->2 |
|  |  |  |  |  |  |  |  | Disable | ->3 |
|  |  |  |  |  |  |  |  | FullCut Mode | ->4 |
|  |  |  |  |  |  |  |  | PartialCutMode | ->5 |
|  |  |  |  |  |  |  |  | Default CutMode | ->6 |
|  |  |  |  |  |  | Auto Cut Settings | ->3 | AUTOCUTSETTINGS:Nouse this function |  |
|  |  |  |  |  |  |  |  | BackToLastMenu | ->1 |
|  |  |  |  |  |  |  |  | Cut paperwhencoveris closed is closed | ->2 |
|  |  |  |  |  |  |  |  | Nocut paperwhen coveris closed | ->3 |
|  |  |  |  |  |  |  |  | Cutpaperwhen poweron | ->4 |
|  |  |  |  |  |  |  |  | Nocut paperwhen poweron | ->5 |
|  |  |  |  |  |  |  |  | Disable | ->6 |
|  |  |  |  | Buzzer | ->4 | BUZZER:Enabled |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Enable | ->2 |  |  |
|  |  |  |  |  |  | Disable | ->3 |  |  |
|  |  | Print Settings | ->5 | PRINTSETTINGS |  |  |  |  |  |
|  |  |  |  | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | Darkness Settings | ->2 | DARKNESS SETTING: Normal |  |  |  |
|  |  |  |  |  |  | Back To Last Menu | ->1 |  |  |
|  |  |  |  |  |  | Low | ->2 |  |  |
|  |  |  |  |  |  | Normal | ->3 |  |  |
|  |  |  |  |  |  | High | ->4 |  |  |
|  |  |  |  |  |  | ExtraHigh | ->5 |  |  |
|  |  |  |  | Paper RollWidth | ->3 | PAPER ROLL WIDTH:80.0mm |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | 57.5mm | ->2 |  |  |
|  |  |  |  |  |  | 80.0mm | ->3 |  |  |
|  |  |  |  |  |  | 82.5mm | ->4 |  |  |
|  |  |  |  | Left Margin | ->4 | LEFTMARGIN:7mm |  |  |  |
|  |  |  |  |  |  | Back To Last Menu | ->1 |  |  |
|  |  |  |  |  |  | 0mm | ->2 |  |  |
|  |  |  |  |  |  | 1mm | ->3 |  |  |
|  |  |  |  |  |  | 3mm | ->4 |  |  |
|  |  |  |  |  |  | 5mm | ->5 |  |  |
|  |  |  |  |  |  | 7mm | ->6 |  |  |
|  |  |  |  |  |  | 9mm | ->7 |  |  |
|  |  |  |  | RightMargin | ->5 | RIGHTMARGIN:9mm |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | 0mm | ->2 |  |  |
|  |  |  |  |  |  | 1mm | ->3 |  |  |
|  |  |  |  |  |  | 3mm | ->4 |  |  |
|  |  |  |  |  |  | 5mm | ->5 |  |  |
|  |  |  |  |  |  | 7mm | ->6 |  |  |
|  |  |  |  |  |  | 9mm | ->7 |  |  |
|  |  |  |  | CR Command | ->6 | CR COMMAND: Disable |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Enable | ->2 |  |  |
|  |  |  |  |  |  | Disable | ->3 |  |  |
|  |  |  |  | CodePage | ->7 | CODEPAGESETTING |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Print All Codepages | ->2 |  |  |
|  |  |  |  |  |  | Select A Codepage | ->3 |  |  |
|  |  |  |  | SavePaper Level | ->8 | SAVEPAPERLEVEL |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Disable | ->2 |  |  |
|  |  |  |  |  |  | 25% | ->3 |  |  |
|  |  |  |  |  |  | 50% | ->4 |  |  |
|  |  |  |  |  |  | 75% | ->5 |  |  |
|  |  |  |  |  |  | 100% | ->6 |  |  |
|  |  | PaperSensor Settings | ->6 | PAPER NEAREND SETTINGS |  |  |  |  |  |
|  |  |  |  | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | PaperLowAlarm | ->2 | PAPERLOWALARM: Enable |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Enable | ->2 |  |  |
|  |  |  |  |  |  | Disable | ->3 |  |  |
|  |  |  |  | StopPrintWhen PAPERLow | ->3 | STOP PRINT WHEN PAPERLOW: Disable |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Enable | ->2 |  |  |
|  |  |  |  |  |  | Disable | ->3 |  |  |
|  |  |  |  | Paper Near End Sensor Sensor | ->4 | PAPER NEAREND SENSER: Enabled |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Enable | ->2 |  |  |
|  |  |  |  |  |  | Disable | ->3 |  |  |
|  |  | Set DefaultConfig | ->7 | SETDEFAULT CONFIGURATION |  |  |  |  |  |
|  |  |  |  | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | Set PrinterTo Default Configuration | ->2 |  |  |  |  |
|  |  | FONTA/FONTB Settings | ->8 | CurrentFont:FONTA |  |  |  |  |  |
|  |  |  |  | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | Select FONTA | ->2 |  |  |  |  |
|  |  |  |  | Select FONTB | ->3 |  |  |  |  |
|  |  |  |  | Select UDFONTA | ->4 |  |  |  |  |
|  |  |  |  | Select UDFONTB | ->5 |  |  |  |  |
|  |  | Beep settings | ->9 | Beepsettings:Disabled |  |  |  |  |  |
|  |  |  |  | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | EnableExternal Herald | ->2 |  |  |  |  |
|  |  |  |  | EnableInternal buzzer | ->3 | BEEPMODE |  |  |  |
|  |  |  |  |  |  | BackToLast Menu | ->1 |  |  |
|  |  |  |  |  |  | Mode1 | ->2 |  |  |
|  |  |  |  |  |  | Mode2 | ->3 |  |  |
|  |  |  |  |  |  | Mode3 | ->4 |  |  |
|  |  |  |  |  |  | Mode4 | ->5 |  |  |
|  |  |  |  |  |  | Mode5 | ->6 |  |  |
|  |  |  |  | AllBeepdisabled | ->4 |  |  |  |  |
|  |  | TestPage Settings | ->10 | TESTPAGEUPTODOWN PRINT:Disabled |  |  |  |  |  |
|  |  |  |  | BackToLast Menu | ->1 |  |  |  |  |
|  |  |  |  | Enable | ->2 |  |  |  |  |
|  |  |  |  | Disable | ->3 |  |  |  |  |
|  |  | Entercode,thenhold Button Down at least 1secondto validate |  |  |  |  |  |  |  |
| SensorTest | ->4 | SensorTestMode: ERROR LED statewillchangeaccordingtosensorstate ToEXIT,holdbuttondown at least 1second |  |  |  |  |  |  |  |
| Print NV Bitmap | ->5 |  |  |  |  |  |  |  |  |
| CutterTest | ->6 |  |  |  |  |  |  |  |  |
| Print Statistics | ->7 | BTP-S80STATISTICS |  |  |  |  |  |  |  |
|  |  | CUT | :0 |  |  |  |  |  |  |
|  |  | TCUT | :0 |  |  |  |  |  |  |
|  |  | LFS |  |  |  |  |  |  |  |
|  |  | TLFS |  |  |  |  |  |  |  |
|  |  | ONTIME | :0 |  |  |  |  |  |  |
| E05Configuration | ->8 | E05CONFIGURATION |  |  |  |  |  |  |  |
|  |  | Reset JK-E04 Config | ->1 |  |  |  |  |  |  |
|  |  | Print Settings | ->2 | IPAddress: MACAddress: SUBNETMask: GATEWAY: Print Port: |  |  |  |  |  |